Schedule of Requirements

STATEMENT OF NEED:

Technical Specifications for Goods:

| Item No | Minimum Technical Requirements | Unit | Quantity |
| --- | --- | --- | --- |
| 1 | 4 MP Fixed Network Camera | Each | 17 |
| 2 | 8 MP IR Fixed Dome Network Camera | Each | 16 |
| 3 | 4 MP IR Fixed Bullet Network Camera | Each | 3 |
| 4 | NVR 32-ch | Each | 2 |
| 5 | U/UTP Cat6 PVC 23 AWG Cable | Each | 6 |
| 6 | 8 TB Purple Surveillance Hard Drive | Each | 8 |
| 7 | Installation, configuration, testing, training and documentation | Each | 1 |

Item No (1): 4 MP Fixed Network Camera

| Camera |  |
| --- | --- |
| Image Sensor | 1/2.8" Progressive Scan CMOS |
| Min. Illumination | Color: 0.001 Lux @ (F1.0, AGC ON) |
| Shutter Speed | 1/3 s to 1/100,000 s |
| Angle Adjustment | Pan: 0° to 360°, tilt: 0° to 75°, rotate: 0° to 360° |
| Slow Shutter | Yes |
| P/N | P/N |
| Wide Dynamic Range | Digital WDR |
| Lens |  |
| Lens Type & FOV | 2.8 mm, horizontal FOV 107°, vertical FOV 56°, diagonal FOV 127° 4 mm, horizontal FOV 84°, vertical FOV 45°, diagonal FOV 99° |
| Aperture | F1.0 |
| Lens Mount | M12 |
| Aperture Type | Fixed |
| Depth of Focus | 2.8 mm: 1.7 m to ∞ 4 mm: 2.1 m to ∞ |
| Illuminator |  |
| Smart Supplement Light | Yes |
| Supplement Light | White |
| White Light Range | Up to 30 m |
| Video |  |
| Main Stream | 50Hz: 25fps (1920 × 1080, 1280 × 960, 1280 × 720) 60Hz: 30fps (1920 × 1080, 1280 × 960, 1280 × 720) |
| Sub Stream | 50Hz: 25fps (640 × 480, 640 × 360, 320 × 240) 60Hz: 30fps (640 × 480, 640 × 360, 320 × 240) |
| Video Compression | Main stream: H.265+/H.265/H.264+/H.264 Sub-stream: H.265/H.264/MJPEG |
| Video Bit Rate | 32 Kbps to 8 Mbps |
| H.264 Type | BaseLine Profile/Main Profile/High Profile |
| H.265 Type | Main Profile |
| Region of Interest (ROI) | 1 fixed region for main stream |
| Network |  |
| Simultaneous Live View | Up to 6 channels |
| API | ONVIF (PROFILE S, PROFILE G), ISAPI, SDK |
| Protocols | TCP/IP, ICMP, HTTP, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, NTP, UPnP, SMTP, IGMP, QoS, IPv6, UDP, Bonjour, SSL/TLS |
| User/Host | Up to 32 users. 3 user levels: administrator, operator and user |
| Security | Password protection, complicated password, HTTPS encryption, IP address filter, security audit log, basic and digest authentication for HTTP/HTTPS, TLS 1.2, WSSE and digest authentication for ONVIF |
| Client | iVMS-4200, Hik-Connect, Hik-Central |
| Web Browser | Plug-in required live view: IE8, IE9, IE10, IE11 Local service: Chrome 57.0+, Firefox 52.0+ |
| Image |  |
| Day/Night Switch | Day, Night, Auto, Schedule |
| Image Enhancement | BLC, HLC, 3D DNR |
| Image Settings | Rotate mode, saturation, brightness, contrast, sharpness, gain, white balance adjustable by client software or web browser |
| Interface |  |
| Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| Intelligent (Traditional Algorithm) |  |
| Basic Event | Motion detection, exception (illegal login) |
| General |  |
| Linkage Method | Upload to FTP/NAS, notify surveillance center, send email, trigger recording, trigger capture *Note: External device is required. |
| Web Client Language | English |
| General Function | Anti- flicker, heartbeat, mirror, privacy masks, flash log, password reset via e-mail, pixel counter |
| Software Reset | Reset via web browser and client software |
| Power Supply | 12 VDC ± 25%, PoE: 802.3af, Type 1 Class 3, reverse polarity protection |
| Power Consumption and Current | 12 VDC, 0.30 A, max. 3.6 W PoE (802.3af, 36 V to 57 V), 0.20 A to 0.10 A, max. 4.5W |
| Power Interface | Ø5.5 mm coaxial power plug |
| Material | Main body: metal; enclosure and base: plastic |
| Camera Dimension | Ø110 × 100.8 mm (4.3" × 4.0") |
| Package Dimension | 150 × 150 × 141 mm (5.9" × 5.9" × 5.6") |
| Camera Weight | Approx. 350 g (0.7 lb.) |
| With Package Weight | Approx. 500 g (1.1 lb.) |
| Approval |  |
| EMC | FCC (47 CFR Part 15, Subpart B); CE-EMC (EN 55032: 2015, EN 61000-3-2: 2014, EN 61000-3-3: 2013, EN 50130-4: 2011 +A1: 2014); RCM (AS/NZS CISPR 32: 2015); IC (ICES-003: Issue 6, 2016); KC (KN 32: 2015, KN 35: 2015) |
| Safety | UL (UL 60950-1); CB (IEC 60950-1:2005 + Am 1:2009 + Am 2:2013); CE-LVD (EN 60950-1:2005 + Am 1:2009 + Am 2:2013); BIS (IS 13252(Part 1):2010+A1:2013+A2:2015) |
| Environment | CE-RoHS (2011/65/EU); WEEE (2012/19/EU); Reach (Regulation (EC) No 1907/2006) |
| Protection | Ingression protection: IP67 (IEC 60529-2013) |

Item No (2): 8 MP IR Fixed Dome Network Camera

| Camera |  |
| --- | --- |
| Image Sensor | 1/2.5" Progressive Scan CMOS |
| Min. Illumination | Color: 0.01 lux @(F1.2, AGC ON), 0.028 lux @(F2.0, AGC ON), 0 lux with IR |
| Shutter Speed | 1/3 s to 1/100,000 s |
| Slow Shutter | Yes |
| Day & Night | IR Cut Filter |
| Digital Noise Reduction | 3D DNR |
| WDR | 120dB |
| 3-Axis Adjustment | Pan: 0° to 355°, tilt: 0° to 75°, rotate: 0° to 355° |
| Lens |  |
| Focal Length | 2.8/4/6/8 mm |
| Aperture | F2.0 |
| Focus | Fixed |
| FOV | 2.8 mm, horizontal FOV: 102°, vertical FOV: 53°, diagonal FOV: 124° 4 mm, horizontal FOV: 79°, vertical FOV: 42°, diagonal FOV: 95° 6 mm, horizontal FOV: 50°, vertical FOV: 28°, diagonal FOV: 59° 8 mm, horizontal FOV: 40°, vertical FOV: 25°, diagonal FOV: 53° |
| Lens Mount | M12 |
| IR |  |
| IR Range | Up to 30 m |
| Wavelength | 850nm |
| Compression: Standard |  |
| Video Compression | Main stream: H.265/H.264 Sub-stream: H.265/H.264/MJPEG Third stream: H.265/H.264 |
| H.264 Type | Main Profile/High Profile |
| H.264+ | Main stream supports |
| H.265 Type | Main Profile |
| H.265+ | Main stream supports |
| Video Bit Rate | 32 Kbps to 16 Mbps |
| Audio Compression (-S) | G722.1/G.711/G726/MP2L2/PCM/MP3 |
| Audio Bit Rate (-S) | 64Kbps(G.711)/16Kbps(G.722.1)/16Kbps(G.726)/32-192Kbps(MP2L2)/8Kbps-320Kbps(MP3) |
| Smart Feature-set |  |
| Smart Event | Line crossing detection, intrusion detection, face detection |
| Basic Event | Motion detection, video tampering alarm, exception (network disconnected, IP address conflict, illegal login, HDD full, HDD error) |
| Linkage Method | Trigger recording: memory card, network storage, pre-record and post-record Trigger captured pictures uploading: FTP, HTTP, NAS, Email Trigger notification: HTTP, ISAPI, alarm output (-S), Email |
| Region of Interest | Support 1 fixed region for main stream and sub-stream |
| Image |  |
| Max. Resolution | 3840 × 2160 |
| Main Stream | 50Hz: 12.5 fps (3840 × 2160), 20fps (3072 × 1728), 25 fps (2560 × 1440, 1920 × 1080, 1280 × 720) 60Hz: 15 fps (3840 × 2160), 20fps (3072 × 1728), 30 fps (2560 × 1440, 1920 × 1080, 1280 × 720) |
| Sub-Stream | 50Hz: 25fps (640 × 480, 640 × 360, 320 × 240) 60Hz: 30fps (640 × 480, 640 × 360, 320 × 240) |
| Third Stream | 50Hz: 25fps (1280 × 720, 640 × 360, 352 × 288) 60Hz: 30fps (1280 × 720, 640 × 360, 352 × 240) |
| Image Enhancement | BLC/3D DNR/HLC |
| Image Setting | Saturation, brightness, contrast, sharpness, AGC, and white balance adjustable by client software or web browser |
| Target Cropping | No |
| Day/Night Switch | Day/Night/Auto/Schedule/Triggered by Alarm In (-S) |
| *Note: When the main stream resolution is 3840 × 2160, max frame rate is 12.5 fps (50Hz)/15 fps (60Hz) for all streams. When the main stream resolution is 3072 × 1728, max frame rate is 20 fps for all streams. |  |
| Network |  |
| Network Storage | Support Micro SD/SDHC/SDXC card (128G), local storage and NAS (NFS,SMB/CIFS), ANR |
| Protocols | TCP/IP, UDP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, PPPoE, NTP, UPnP, SMTP, SNMP, IGMP, 802.1X, QoS, IPv6, UDP, Bonjour, SSL/TLS |
| General Function | Anti-flicker, three streams, heartbeat, mirror, privacy masks, password reset via e-mail, pixel counter, HTTP listening |
| API | ONVIF (PROFILE S, PROFILE G, PROFILE T), ISAPI, SDK |
| Security | Password protection, complicated password, HTTPS encryption, 802.1X authentication (EAP-TLS 1.2, EAP-LEAP, EAP-MD5), watermark, IP address filter, basic and digest authentication for HTTP/HTTPS, WSSE and digest authentication for ONVIF, TLS1.2 |
| Simultaneous Live View | Up to 6 channels |
| User/Host | Up to 32 users 3 levels: Administrator, Operator and User |
| Client | iVMS-4200, Hik-Connect, Hik-Central |
| Web Browser | Plug-in required live view: IE8+ Plug-in free live view: Chrome 57.0+, Firefox 52.0+, Safari 11+ Local Service: Chrome 41.0+, Firefox 30.0+ |
| Interface |  |
| Audio (-S) | 1 input (line in/mic in), 1 output (line out), terminal block, mono sound |
| Alarm (-S) | 1 input, 1 output (max. 12 VDC, 30 mA), terminal block |
| Video Output | No |
| Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| On-board Storage | Built-in microSD/SDHC/SDXC slot, up to 128 GB |
| SVC | H.264 and H.265 encoding support |
| Reset Button | Yes |
| Audio |  |
| Environment Noise Filtering | Yes |
| Audio Sampling Rate | 8 kHz/16 kHz/32 kHz/44.1 kHz/48 kHz |
| General |  |
| Operating Conditions | -30 °C to +60 °C (-22 °F to +140 °F), humidity 95% or less (non-condensing) |
| Web Client Language | 32 languages English, Russian, Estonian, Bulgarian, Hungarian, Greek, German, Italian, Czech, Slovak, French, Polish, Dutch, Portuguese, Spanish, Romanian, Danish, Swedish, Norwegian, Finnish, Croatian, Slovenian, Serbian, Turkish, Korean, Traditional Chinese, Thai, Vietnamese, Japanese, Latvian, Lithuanian, Portuguese (Brazil) |
| Power Supply | 12 VDC ± 25%, Φ 5.5 mm coaxial power plug PoE(802.3af, class 3) |
| Power Consumption and | DC: 12 V, 0.6 A, max. 7.5 W |
| Current | PoE: (802.3af, 36 V to 57 V), 0.3 A to 0.2 A, max. 9 W |
| Protection Level | IP67, IK10 |
| Material | Camera body: metal, bubble: plastic |
| Dimensions | Camera: Φ 111 mm × 82.4 mm (Φ 4.4" × 3.2") Package: 134 × 134 × 108 mm (5.3" × 5.3" × 4.3") |

Item No (3): 4 MP IR Fixed Bullet Network Camera

| Camera |  |
| --- | --- |
| Image Sensor | 1/3" Progressive Scan CMOS |
| Min. Illumination | Color: 0.01 Lux @ (F1.2, AGC ON), 0.018 Lux @ (F1.6, AGC ON), 0 Lux with IR |
| Shutter Speed | 1/3 s to 1/100,000 s |
| Slow Shutter | Yes |
| Day & Night | IR Cut Filter |
| Digital Noise Reduction | 3D DNR |
| WDR | 120dB |
| 3-Axis Adjustment | Pan: 0° to 360°, tilt: 0° to 100°, rotate: 0° to 360° |
| Lens |  |
| Focal length | 2.8/4/6/8 mm |
| Aperture | F1.6 |
| Focus | Fixed |
| FOV | 2.8 mm, horizontal FOV: 103°, vertical FOV: 58°, diagonal FOV: 123° 4 mm, horizontal FOV: 83°, vertical FOV: 45°, diagonal FOV: 99° 6 mm, horizontal FOV: 51°, vertical FOV: 29°, diagonal FOV: 58° 8 mm, horizontal FOV: 39°, vertical FOV: 22°, diagonal FOV: 45° |
| Lens Mount | M12 |
| IR |  |
| IR Range | -I5: Up to 50 m -I8: Up to 80 m |
| Wavelength | 850nm |
| Compression Standard |  |
| Video Compression | Main stream: H.265/H.264 Sub-stream: H.265/H.264/MJPEG Third stream: H.265/H.264 |
| H.264 Type | Main Profile/High Profile |
| H.264+ | Main stream supports |
| H.265 Type | Main Profile |
| H.265+ | Main stream supports |
| Video Bit Rate | 32 Kbps to 16 Mbps |
| Smart Feature-set |  |
| Smart Event | Line crossing detection, intrusion detection, face detection |
| Basic Event | Motion detection, video tampering alarm, exception (network disconnected, IP address conflict, illegal login, HDD full, HDD error) |
| Linkage Method | Trigger recording: memory card, network storage, pre-record and post-record Trigger captured pictures uploading: FTP, HTTP, NAS, Email Trigger notification: HTTP, ISAPI, Email |
| Region of Interest | 1 fixed region for main stream and sub-stream |
| Image |  |
| Max. Resolution | 2688 × 1520 |
| Main Stream | 50Hz:25fps (2688 × 1520, 2560 × 1440, 2304 × 1296, 1920 × 1080, 1280 × 720) 60Hz: 30fps (2688 × 1520, 2560 × 1440, 2304 × 1296, 1920 × 1080, 1280 × 720) |
| Sub-Stream | 50Hz: 25fps (640 × 480, 640 × 360, 320 × 240) 60Hz: 30fps (640 × 480, 640 × 360, 320 × 240) |
| Third Stream | 50Hz: 25fps (1280 × 720, 640 × 360, 352 × 288) 60Hz: 30fps (1280 × 720, 640 × 360, 352 × 240) |
| Image Enhancement | BLC/3D DNR/HLC |
| Image Setting | Rotate mode, saturation, brightness, contrast, sharpness, AGC, and white balance are adjustable by client software or web browser |
| Target Cropping | No |
| Day/Night Switch | Day/Night/Auto/Schedule |
| Network |  |
| Network Storage | Support Micro SD/SDHC/SDXC card (128G), local storage and NAS (NFS,SMB/CIFS), ANR |
| Protocols | TCP/IP, UDP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, PPPoE, NTP, UPnP, SMTP, SNMP, IGMP, 802.1X, QoS, IPv6, UDP, Bonjour, SSL/TLS |
| General Function | Anti-flicker, three streams, heartbeat, mirror, privacy masks, password reset via e-mail, pixel counter, HTTP listening |
| Security | Password protection, complicated password, HTTPS encryption, 802.1X authentication (EAP-TLS 1.2, EAP-LEAP, EAP-MD5), watermark, IP address filter, basic and digest authentication for HTTP/HTTPS, WSSE and digest authentication for ONVIF, TLS1.2 |
| API | ONVIF (PROFILE S, PROFILE G, PROFILE T), ISAPI, SDK |
| Simultaneous Live View | Up to 6 channels |
| User/Host | Up to 32 users 3 levels: Administrator, Operator and User |
| Client | iVMS-4200, Hik-Connect, Hik-Central |
| Web Browser | Plug-in required live view: IE8+ Plug-in free live view: Chrome 57.0+, Firefox 52.0+, Safari 11+ Local Service: Chrome 41.0+, Firefox 30.0+ |
| Interface |  |
| Communication Interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| Video Output | No |
| On-board storage | Built-in Micro SD/SDHC/SDXC slot, up to 128 GB |
| SVC | H.264 and H.265 encoding |
| Reset Button | Yes |
| General |  |
| Operating Conditions | -30 °C to +60 °C (-22 °F to +140 °F), Humidity 95% or less (non-condensing) |
| Web Client Language | 32 languages English, Russian, Estonian, Bulgarian, Hungarian, Greek, German, Italian, Czech, Slovak, French, Polish, Dutch, Portuguese, Spanish, Romanian, Danish, Swedish, Norwegian, Finnish, Croatian, Slovenian, Serbian, Turkish, Korean, Traditional Chinese, Thai, Vietnamese, Japanese, Latvian, Lithuanian, Portuguese (Brazil) |
| Power Supply | 12 VDC ± 25%, Φ 5.5 mm coaxial power plug PoE (802.3af, class 3) |
| Power Consumption and Current | -I5: 12 VDC, 0.6 A, max. 7.5 W PoE (802.3af, 36 V to 57 V), 0.2 A to 0.1 A, max. 9.5 W -I8: 12 VDC, 0.95 A, max. 11.5 W PoE (802.3af, 36 V to 57 V), 0.35 A to 0.1 A, max. 12.5 W |
| Protection Level | IP67 |
| Material | Metal |
| Dimensions | Camera: Φ 105 × 299.7 mm (Φ 4.1" × 11.8") Package: 386 × 156 × 155 mm (15.2" × 6.1" × 6.1") |
| Weight | Camera: Approx. 1180 g (2.6 lb.) With Package: Approx. 1700 g (3.7 lb.) |

Item No (4): NVR 32 channels

| Video/Audio input | IP video input | 32-ch |
| --- | --- | --- |
|  |  | Up to 12 MP resolution |
|  | Two-way audio | 1-ch, RCA (2.0 Vp-p, 1 k Ω) |
| Network | Incoming bandwidth | 320 Mbps |
|  |  | or 200 Mbps (when RAID is enabled) |
|  | Outgoing bandwidth | 256 Mbps, or 200 Mbps (when RAID is enabled) |
|  | Remote connection | 128 |
| Video/Audio output | Recording resolution | 12 MP/8 MP/6 MP/5 MP/4 MP/3 MP/1080p/UXGA/720p/VGA/4CIF/DCIF /2CIF/CIF/QCIF |
|  | CVBS output (Optional) | 1-ch, BNC (1.0 Vp-p, 75 Ω), resolution: PAL: 704 × 576, NTSC: 704 × 480 |
|  | VGA1 /HDMI1 output resolution | HDMI1: 4K (3840 × 2160)/60Hz, 4K (3840 × 2160)/30Hz, 2K (2560 × 1440)/60Hz, 1920 × 1080p/60Hz, 1600 × 1200/60Hz, 1280 × 1024/60Hz, 1280 × 720/60Hz, 1024 × 768/60Hz |
|  |  | VGA1: 2K (2560 × 1440)/60Hz, 1920 × 1080p/60Hz, 1600 × 1200/60Hz, 1280 × 1024/60Hz, 1280 × 720/60Hz, 1024 × 768/60Hz |
|  | VGA2 /HDMI2 output resolution | 1920 × 1080p/60Hz, 1280 × 1024/60Hz, 1280 × 720/60Hz, 1024 × 768/60Hz |
|  | Audio output | 2-ch, RCA (2.0Vp-p, 1 KΩ) |
| Decoding | Decoding format | H.265/H.265+/H.264/H.264+/MPEG4 |
|  | Live view/Playback resolution | 12 MP/8 MP/6 MP/5 MP/4 MP/3 MP/1080p/UXGA/720p/VGA/4CIF/DCIF /2CIF/CIF/QCIF |
|  | Synchronous playback | 16-ch |
|  | Capability | 2-ch @ 12 MP (20fps) / 4-ch @ 8 MP (25fps) / 8-ch @ 4MP (30fps) / 16-ch @ 1080p (30fps) |
| Network management | Network protocols | TCP/IP, DHCP, Hik-Connect, DNS, DDNS, NTP, SADP, SMTP, NFS, iSCSI, UPnP™, HTTPS |
| Hard disk | SATA | 8 SATA interfaces |
|  | eSATA | 1 eSATA interface |
|  | Capacity | Up to 10TB capacity for each HDD |
| Disk array | Array type | RAID0, RAID1, RAID5, RAID6, RAID10 |
|  | Number of arrays | 4 |
| External interface | Network interface | 2, RJ-45 10/100/1000 Mbps self-adaptive Ethernet interface |
|  | Serial interface | RS-232; RS-485; Keyboard |
|  | USB interface | Front panel: 2 × USB 2.0; Rear panel: 1 × USB 3.0 |
|  | Alarm in/out | 16/4 |
| General | Power supply | 100 to 240 VAC, 50 to 60 Hz |
|  | Max. Power | 200 W |
|  | Consumption (without hard disk) | ≤ 30 W |
|  | Working temperature | -10 °C to 55 °C (14 °F to 131 °F) |
|  | Working humidity | 10 to 90 % |
|  | Dimensions(W × D × H) | 445 × 470 ×90 mm (17.5"× 18.5" × 3.5") |
|  | Weight (without hard disk) | ≤ 10 kg (22 lb) |

Item (5): U/UTP Cat6 PVC 23 AWG Cable

| Category | Cat.6 |  |
| --- | --- | --- |
| Sheath printing | HIKVISION 4PR 23AWG 75°C U/UTP CAT6 VERIFIED TO ANSI/TIA-568-C.2 PVC ****m mm/dd/yyyy Tracing Code-HS |  |
| Reference standard | ISO/IEC11801, ANSI/TIA-568-C.2, RoHS 2.0 |  |
| Fire resistance | No |  |
| Conductor | Material | Solid-Bare Copper (99.95%) |
|  | Diameter | 0.55 ± 0.005 mm, 23 AWG |
| Insulation | Material | HDPE |
|  | Diameter | 0.98 ± 0.05 mm |
| Sheath | Thickness | 0.50 ± 0.10 mm |
|  | Diameter | 6.00 ± 0.30 mm |
|  | Material | PVC |
|  | Color | Orange |
| Rip-cord | Yes |  |
| Separator | Yes |  |
| Sheath physical properties | Before aging | Tensile strength ≥ 13.5 Mpa Elongation ≥ 150% |
|  | Aging period | 100°C × 24h × 10d |
|  | After aging | Tensile strength ≥ 12.5 Mpa Elongation ≥ 125% |
|  | Cold bend | -20 ± 2°C × 4h, 8 × cable O.D, no visible cracks |
| Electrical Characteristics (20°C) | 1.0-250.0 MHz impedance | 100 ± 15 Ω @ ≤ 100 MHz 100 ± 25 Ω @100-250 MHz |
|  | 1.0-250.0 MHz delay skew | ≤ 45 ns/100 m |
|  | DC resistance | ≤ 9.38 Ω/100 m |
|  | DC conductor resistance unbalance | ≤ 5.0% |
|  | Package dimension | 400 mm (W) × 400 mm (H) × 240 mm (D) (15.75" × 15.75" × 9.45") |
|  | Weight | 12.3 ± 0.5kg |
|  | Cable length | (305 ± 1.5) m |
|  | Ambient temperature | -20°C to 75°C (-4°F to 167°F) |
|  | Operating voltage | 60 V |

Item (6): 8 TB Purple Surveillance Hard Drive

Capacity: 8 TB

Interface: SATA

Form Factor: 3.5 Inch

Disk Speed (RPM): 7200rpm

Compatibility: WD Purple™ surveillance storage is built to handle up to 64 cameras per drive and is designed for 24/7, always on, high-definition surveillance security systems with more than eight bays2.

Dimensions (L x W x H): 5.79" x 4" x 1.03"

Item (7): Installation, configuration, testing, training and documentation

DELIVERY REQUIREMENTS:

| Delivery Requirements |  |
| --- | --- |
| Delivery date and time | Vendor shall deliver the goods and install all required equipment within maximum of 15 days after PO issuance. |
| Delivery Terms (INCOTERMS 2020) | N/A |
| Customs clearance (Must be linked to INCOTERM) | ☒ Not applicable Shall be done by: ☐ Name of organization ☐ Vendor/bidder ☐ Freight Forwarder |
| Exact Address(es) of Delivery Location(s) | Villa number 37, Street number 85, Maadi, Cairo, Egypt |
| Distribution of shipping documents (if using freight forwarder) | N/A |
| Special Packing Requirements | N/A |
| Training on Operations and Maintenance | Required |
| Warranty Period | 1 Year |
| After-sales service and local service support requirements | Not Required |
| Preferred Mode of Transport | Land |
| Subcontracting | The vendor is strongly encouraged not to sub-contract > 50% of the total work. If a vendor/s intend to do so, they must state the actual percentage (that will be outsourced) in their offer/quotation. |

INSPECTIONS AND TESTS:

The following inspections and tests shall be performed:

- Test the items and ensure that all system and its items are properly installed and configured, orientation and written documentation with drawings is needed.

- If the goods fail to meet the laid down specifications, the vendor shall take immediate steps to remedy the deficiency or replace the defective goods to the satisfaction of UN Women.